PROGRAM: IIR				PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: Edward J. Smith

PROGRAM FUNCTION AND METHODS:

IIR is a programmable, n-th order, Butterworth, low-pass, recursive,

infinite impulse response, bidirectional digital filter.  IIR determines

which filters (1-,2-,3-, or 4-pole) to call in order to produce the

desired n-pole filter.  IIR was adapted from the VRTC program IIRFIL

developed by B. Stevens.

PROGRAM FEATURES:

IIR will prompt you for 3 values: frequency cutoff (Hz), attenuation (dB)

at stop band frequency, and stop band frequency (Hz).  From these it will

compute the number of poles according to this formula:

# of poles = -FATT/(20.*LOG10(FSTP/FCUT)) + 0.5

FATT - Frequency attenuation

FSTP - Stop frequency

FCUT - Cutoff frequency

SETUP/RUN INSTRUCTIONS:

An existing UDS time series file must be supplied to the program.  All

entered values must be real (fractions are truncated), and the frequency

attenuation must be negative.

KNOWN PROBLEMS/CONSTRAINTS:

IIR has been observed to move the initial signal tail by large amounts in

some cases.

BWFilt has a 25 percent higher cutoff frequency than IIR, therefore, if

you want to duplicate the response of the BWFilt filter you must raise

IIR's cutoff freq. by 25 percent.

The following is a table that will let you duplicate the filtering process

of BWFilt for these SAE classes:

CLASS      FCUT     FATT     FSTP

60        125      -40      395

100        225      -40      712

180        375      -40     1186

600       1250      -40     3953