PROGRAM: BWFilt				PROGRAM MODIFICATION DATE:	6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	6/95

AUTHOR: ASGI

PROGRAM FUNCTION AND METHODS:

BWFilt performs two pass, zero phase shift, second order

Butterworth filtering of UDS time series files.  Ten point

tail averaging is used for the start-up.

PROGRAM FEATURES/OPTIONS:

Default cutoff frequencies are provided based on current NHTSA

filtering standards.

Default output file names are provided using the current file

naming conventions.

BWFilt has been modified to provide -3 db at cutoff and to meet

the J211 SAE standard (11/83).  This was accomplished by applying

a 1.25 factor to the cutoff value in the FILTER subroutine.

BWFilt has been modified to eliminate initial tail discontinuity

for non-transient signals by using inverted, mirrored values for

the appended initial portion.  The final tail treatment has also

been improved with an inverted, mirrored extrapolation for the

points used to start the second filtering pass.

SETUP/RUN INSTRUCTIONS:

An existing UDS y-series file must be supplied to the program.

KNOWN PROBLEMS/CONSTRAINTS:

The general nature of this filter is that there is some distortion

of the data tail values and integrals.  It is not recommended using

BWFilt for filtering below 100 Hz if such quantitative accuracy is

important.