PROGRAM: UDSTool			PROGRAM MODIFICATION DATE:	6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	6/95

AUTHOR: Stuart G. Mentzer

PROGRAM FUNCTION AND METHODS:

UDSTool performs interactive display, modification, and creation of UDS

files. The measurement data and specifications of a UDS file can be

viewed and changed.

PROGRAM FEATURES/OPTIONS:

The display options in UDSTool include an overview or table of the

signal and its relevant integrals/derivatives or display of the UDS

specification fields. An x-axis window can be applied to the overview

and table displays and subsampling is available for the table listing.

Acceleration integrals use the INIVEL value for the time-zero

velocity; other integrals use zero for the initial value.

Displacement derivatives use INIVEL for the time-zero linear velocity.

Output from any of these display options can be optionally routed to a

file.

The modifications which can be performed with UDSTool include

calibration, bias, interpolation, value, domain, and specification

changes.  The calibration and bias changes are segmented linear

functions over the even x-axis or array index step.  Individual data

values can be changed or a UDS or ASCII file can be read to replace the

entire data set.

Output of the data in its current state can be performed at any point

in the process.

A new UDS file can be created from scratch by entering a name of a

nonexistent UDS file. Default values for key fields are provided and

can be altered. Other specification fields can then be entered by

using the Modify|Specs option. Measurement data can be read from a

UDS or ASCII file (the type of file is determined automatically).

ASCII measurement data files may have a non-numeric header of arbitrary

length followed by columns of numbers.  UDSTool analyzes the column

structure of the first numeric record and prompts the user for the

column to read for the Y and, for X-Y files, X data.

Defaults are provided for most of the user prompts. Verification for

the changes and file I/O are printed to log the operation sequence.

KNOWN PROBLEMS/CONSTRAINTS:

In general, the correction of timing, bias, and calibration errors is

dependent on some supplementary information, such as the assessment of

related measurements (same test and axis), supporting measurements

(such as the pre- and post-test relative displacements of

accelerometers or estimates from another source such as film data), and

the experience of the analyst in identifying various errors.  The VeCor

program performs automated correction of uniaxial test data based on

such information and physical principles.