PROGRAM: Trunc				PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: Stuart G. Mentzer

PROGRAM FUNCTION AND METHODS:

Trunc performs user-requested truncation or extension and subsampling

of UDS files.

PROGRAM FEATURES/OPTIONS:

Either or both data tails can be truncated or extended (with zeros)

and subsampling can be performed.

A report file named trunc.rep is generated if the REP command line

argument is used.

SETUP/RUN INSTRUCTIONS:

An existing UDS time series file must be supplied to the program.

KNOWN PROBLEMS/CONSTRAINTS:

The data array index is limited by the bounds of the Trunc array

program.

Trunc does not perform domain shifting of the pulse (the alignment is

not changed, the tail lengths are just adjusted).  Use UDSTool or other

programs to perform domain shifting.

Subsampling of data is not recommended, in general.  Unless the

sampling frequency is at least twice the stop frequency of the filter

(the Nyquist frequency) information will be lost.  It is better to

filter to an appropriate frequency before increasing the time increment

in the pulse.