PROGRAM: MADY2UDS			PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: NHTSA, NRD-11			DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: S. Summers

PROGRAM FUNCTION AND METHODS:

MADY2UDS converts the time history data from madymo runs into a NHTSA UDS

files.  This program has been tested with the LINACC.DAT and the FORCES.DAT

output files from MADYMO.  The output UDS file contains a

location/measurement from the madymo data file.

PROGRAM FEATURES/OPTIONS:

One MADYMO location/measurement is output per UDS file.  The user is

prompted to specify which location and measurement should be output.  The

program will accept ranges and the * wildcard when specifying the

locations/measurements to be output.  Using the * wildcard means that all

the locations/measurements will be output.  If the the * wildcard is

present anywhere on the input line, then all the locations/measurements

will be output.

There is currently a limit of 20 curves that can be converted in a single pass.

Additional curves can be processed by using multiple passes through the file.

SETUP/RUN INSTRUCTIONS:

The user must supply the name of the MADYMO file, identify the location and

measurement to be output, and supply the name of the output UDS file.

KNOWN PROBLEMS/CONSTRAINTS:

The MADYMO data file must start with time zero.

EXAMPLE:

MADY2UDS

Enter MADYMO File Name, CR or ^Z to end >>  LINACC.DAT

Enter location(s) for UDS Data(ie: 1,2,5-8,...)

1   HEAD

2   SPINE T1

3   UPPER RIB

4   MIDDLE RIB

5   LOWER RIB

6   SPINE T12

7   PELVIS

Location >> 1,2

Enter Measurement(s) for UDS Data (ie: 1,2,5-8...)

1   RES. ACCELERATION (M/S**2)

2   X-COMP. ACCELERATION (M/S**2)

3   Y-COMP. ACCELERATION (M/S**2)

4   Z-COMP. ACCELERATION (M/S**2)

Measurement >> 1

HEAD

RES. ACCELERATION (M/S**2)

UDS File Name : HEAD.ACC

SPINE T1

RES. ACCELERATION (M/S**2)

UDS File Name :  SPINE.ACC

Enter MADYMO File Name, CR or ^Z to end >>