PROGRAM: HIC15				PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: Stuart G. Mentzer

PROGRAM FUNCTION AND METHODS:

HIC15 finds the maximum 15 msec Head Injury Criterion (HIC) value of

a head cg resultant acceleration time seies and the corresponding time

interval. The HIC value is the integral of the acceleration (in G'S)

to the 2.5 power divided by the time interval to the 1.5 power. The

time interval is restricted in HIC15 to be at most 15 msec.

HIC15 uses an efficient branch and bound algorithm which finds the

global maximum while exploiting the monotonicities within the HIC

function. HIC15 is an adaptation of HIC with the additional

constraint on the time interval.

See report DOT-HS-806-681 for a	description of the basic algorithm.

PROGRAM FEATURES/OPTIONS:

A new version of the UDS file with the computed HIC value is output

to the user's account if requested with the UPDATE command line

argument.

SETUP/RUN INSTRUCTIONS:

An existing resultant time series must be supplied to the program.

If the data isn't raw head cg acceleration a warning is produced.

KNOWN PROBLEMS/CONSTRAINTS:

None.